| AREA | FREQUENCY | PRODUCT AND DOSING | TOOLS | METHOD | DOCUMENTS | CLEANER | CHECKER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Removal of waste | Continual | N/A | N/A | By hand to skips | N/A | Labourer | Chargehand |
| Factory floor | Continual | Floor cleaner / degreaser; Dilute 1 : 60 with water | NILFISK BA 500 floor cleaning machine | Machine wash factory floor | N/A | Contractor | Supervisor |
| Packing Tables | Each order | Bactericidal wipes | N/A | Wipe packing table | Production Order HD 1.23 | Operator | Chargehand |
| Machines (light) | Each order | Bactericidal wipes | N/A | Remove all swarf / off cuts and wipe | Production Order HD 1.23 | Operator | Chargehand |
| M/C tool | Tool Change | Bactericidal wipes | N/A | As per document instructions | Eng. Safe sign off HD 16.38 | Engineer | Eng. Mgr. |
| Machines (deep) | Weekly | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths | Remove all dust, swarf & off cuts, clean rollers and wipe machine down | Dept 1 Cleaning Record QD 16.37 | Operator | Chargehand |
| Factory floor (perimeter) | Weekly | Bactericidal cleaner; Dilute 1 : 60 with hot water | Socket brush and socket mop | Pull out pallets brush and mop | Dept 1 Cleaning Record QD 16.37 | Labourer | Chargehand |
| Conveyor | Weekly | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths | Remove tape and wipe | Dept 1 Cleaning Record QD 16.37 | Labourer | Chargehand |
| Wall fans | Weekly | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths, Vacuum if required | Remove dust and fluff and wipe | Dept 1 Cleaning Record QD 16.37 | Labourer | Chargehand |
| Pipe-work, overheads and ledges (within easy reach) | Weekly | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths, Vacuum if required | Remove dust and wipe | Dept 1 Cleaning Record QD 16.37 | Labourer | Chargehand |
| Packaging store cupboards | Quarterly | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths, Vacuum if required | Pull out packaging remove dust and wipe | Dept 1 Cleaning Record QD 16.37 | Labourer | Chargehand |
| Walls and paint-work | Annually | Bactericidal cleaner; Dilute 1 : 100 with hot water | Hand brush, shovel & disposable cloths | Hand wash down, reachable only | N/A | Labourer | Supervisor |
| High level | 3 years | Neutral detergent | Industrial Vacuum | All items above breeze blocks Vacuum & wash | Contract documents | Technical Contractor | Tech. Mgr. |