- HACCP Study

Demonstrates
Senior Management
commitment to HACCP

| Approved By | Managing Director |
| --- | --- |
| Signature |  |
| Position |  |
| Approval Date | 19.11.02 |
| Current Revision | 5 |

PAGE 1 OF 8										            SOP 1.1 Revision 5

Issue date: 19.11.02

- Terms of reference

- The HACCP study covers all products manufactured at -----------------------------------site. 

The study covers the production process from raw material source to delivery of finished product to the customer.  For each process step all potential hazards to consumer safety and product integrity have been identified and appropriate control measures have been established to minimise their risk.

- The food safety hazards considered by the study are microbiological, foreign objects and chemical contamination, as well as packaging defects critical to consumer safety.

All elements of the HACCP study have been carried out by the following HACCP Team:

Mr Brown			-	Technical Manager

Mr White			-	Quality Manager

Mrs Black			-	Print Manager

Mrs Green			-	Conversion Manager

Mr Pink			-	Engineering Manager

The HACCP Team will review the HACCP Study annually and following significant changes to the process.

- Product description and intended use

-------------------------------Ltd manufacture and supply ---------------------------------- which are used by our customers to provide a ----------------------------------- to their products.  Our customers are primarily from the dairy, food, chemical, optical and pharmaceutical industries.

The products are supplied in ----------------------------------form and are made from a variety of materials including---------------------------------. All products are coated on the 'food contact' side with ------------------------------------------.

------------------------------Ltd classify our products as high risk as they are used in direct contact with food and other hygiene sensitive products.  Any contamination of the -------------------------could be transferred to the customers product and pose a risk to end users, who are usually the general public.  It is extremely important therefore that we identify all potential food safety hazards throughout our manufacturing process and implement appropriate control measures.

PAGE 2 OF 8										            SOP 1.1 Revision 5

Issue date: 19.11.02

- Process flow diagram

The process flow diagram details the process steps of the production process from raw material source to delivery of finished product to the customer.

Raw material source / receipt

Storage of raw material

General factory wide contamination risks

Glass & brittle plastic

Blades

Pests

Microbiological

Lubricants & chemicals

Engineering parts

Dust & dirt

Hairs

Blood

Wood

Jewellery

Food stuffs

Off-cuts
Extraneous matter

Unpacked for production

Printed

Stored for conversion

Converted

Stored for despatch

Delivered to customer

PAGE 3 OF 8										                SOP 1.1 Revision 5

Issue date: 19.11.02

- Process flow verification / change log

The HACCP team has agreed the food safety hazards associated with each process step. The HACCP study and the control and monitoring measure will be reviewed at least annually and following a significant process change.  All amendments are recorded on the following verification form.

| Date | Amendment | Signature |
| --- | --- | --- |
| 10.05.00 | Revision 1 – New document |  |
| 25.10.00 | Revision 2 – Inclusion of the following new hazards: - Foreign body a - Foreign body b - Chemical contaminant a |  |
| 25.06.01 | Revision 3 – Inclusion of CCP numbers on process flow chart and reordering of process steps. |  |
| 25.03.02 | Revision 4 – Process flow reviewed and CCP's amended. Addition of risk rating (likelihood/severity) to hazard analysis chart. |  |
| 19.11.02 | Revision 5 – Terms of reference amended to be more clear. Critical limits added to the hazard analysis for CCP's. |  |

PAGE 4 OF 8										           	 SOP 1.1 Revision 5

Issue date: 19.11.02

- Typical food safety hazards

- Microbiological:			Yeasts

Moulds

Enterobacteriaceae

Staphylococcus aureus

E Coli

Listeria

Salmonella

Foreign Object:			Blades

Off-cuts

Hair

Blood

Glass, brittle plastic or ceramic										Wood

Soil

Pests (rodents, mice)

Insects

Jewellery

Food stuffs
					Extraneous matter

Chemical:				Oil

Cleaning materials

Grease

Inks

Solvents

PAGE 5 OF 8										                SOP 1.1 Revision 5

Issue date: 19.11.02

- Hazard analysis - risk to food safety

- L = Likelihood of the Occurrence		S = Severity of the Outcome		R = Risk Level (L x S)

- 3 = High					3 = High				1-3 = Low risk (establish control measures where appropriate)

- 2 = Medium					2 = Medium				4-6 = Medium risk (establish control measures)

1 = Low						1 = Low					7-9 = Substantial risk (Critical Control Point)

|  |  |  | Hazard Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Process Step | Hazard | Type | L | S | R | CCP | Control Measures |  | Documentation | HP 1.3 Hygiene Training procedure, HP 1.13 GMP/GHP audit procedure |
|  | Analysed in the table are the generic contamination hazards that may be present throughout the manufacturing process. The hazards detailed are not process specific and are controlled by essential factory wide procedures. | Glass and brittle plastics | F | 1 | 3 | 3 | No | Minimise glass in the factory and audit. Ensure glass breakage action and investigation systems are in place |  | HP 1.5 Glass and brittle plastic control procedure |  |
|  |  | Blades | F | 1 | 3 | 3 | No | System for safe issue, return and disposal of blades. Ensure blade loss/damage action and investigation systems are in place |  | HP 1.6 Blade control procedure |  |
|  |  | Pests | F,M | 1 | 3 | 3 | No | Ensure pest control systems are in place |  | HP 1.8 Pest control procedure |  |
|  |  | Microbiological | M | 1 | 3 | 3 | No | Clearly communicate personal hygiene rules and ensure hygienic conditions are monitored by audits |  | Hygiene Code of Practice QD 16.23 HP 1.12 Microbiological audit procedure |  |
|  |  | Dust and dirt | F | 2 | 1 | 2 | No | Ensure all cleaning requirements are specified and are carried out as scheduled |  | HP 1.7 Cleaning procedure |  |
|  |  | Wood splinters | F | 1 | 2 | 2 | No | Exclude wood from production areas |  | N/A |  |
|  |  | Hairs | F | 2 | 1 | 2 | No | Ensure hairnets and beard snoods are worn |  | Hygiene Code of Practice QD 16.23 |  |
|  |  | Blood | F | 2 | 1 | 2 | No | Ensure accidents are reported immediately and cuts are suitably protected |  | Hygiene Code of Practice QD 16.23 |  |
|  |  | Foodstuffs | F | 2 | 1 | 2 | No | Restrict food and drink to designated areas outside of production |  | Hygiene Code of Practice QD 16.23 |  |
|  |  | Jewellery and personal items | F | 1 | 2 | 2 | No | Restrict wearing of jewellery and bringing of personal items into production areas |  | Hygiene Code of Practice QD 16.23 |  |

CCP = Critical Control Point 		F = Foreign Body hazard		M = Microbiological hazard		C = Chemical hazard

PAGE 6 OF 8				 													SOP 1.1 Revision 5

- Issue date: 19.11.02

|  |  |  | Hazard Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Process Step | Hazard | Type | L | S | R | CCP | Control Measures |  | Documentation | HP 1.3 Hygiene Training procedure, HP 1.13 GMP/GHP audit procedure |
|  | Raw material source / receipt | Raw material not approved for direct food contact | C | 1 | 3 | 3 | No | Ensure that all raw material is purchased approved for 'direct food contact' and the supplier provides all relevant specifications and certification |  | QP 3.1 Raw material assessment and approval procedure HP 1.9 Food contact compliance procedure |  |
|  |  | Generic contamination caused by poor handling and hygiene controls at suppliers | F,M,C | 1 | 2 | 2 | No | Only purchase raw material from approved suppliers who can demonstrate the required level of hygiene controls. Approval Questionnaire and auditing |  | QP 5.1 Selection and assessment of raw material suppliers procedure QP 11.2 Supplier audit procedure. |  |
|  |  | Contaminated deliveries | F,M,C | 1 | 2 | 2 | No | Ensure all deliveries of lidding material are inspected for odour, damp or contamination |  | QP 6.1 Receipt of material procedure (incoming vehicle and load inspection) |  |
|  | Process Step | Hazard | Type | L | S | R | CCP | Control Measures |  | Documentation |  |
|  | Storage of raw material | Generic contamination caused by poor handling and hygiene controls in Stores | F,M,C | 1 | 2 | 2 | No | Ensure that all raw material remains in original packaging until issued to production. Ensure that the stores department cleaning schedule is applied and external doors are kept closed when not in use |  | HP 1.7 Cleaning procedure Hygiene Code of Practice QD 16.23 |  |
|  | Process Step | Hazard | Type | L | S | R | CCP | Control Measures |  | Documentation |  |
|  | Unpacked for production | Generic contamination caused by poor handling and hygiene controls in Stores | F,M,C | 1 | 2 | 2 | No | Ensure that all raw material is inspected for odour, damp or contamination when unpacking and is stored on clean plastic pallets for issue to production Ensure that all personal hygiene rules are applied |  | HP 1.7 Cleaning procedure Hygiene Code of Practice QD 16.23 |  |

- CCP = Critical Control Point 		F = Foreign Body hazard		M = Microbiological hazard		C = Chemical hazard

PAGE 7 OF 8				 													SOP 1.1 Revision 5

- Issue date: 19.11.02

- Complete hazard analysis for each process step

- HMS 1 - Hygiene Management System procedures

| HP 1.1 Hazard analysis procedure |  |
| --- | --- |
| HP 1.2 Medical review procedure |  |
| HP 1.3 Hygiene training procedure |  |
| HP 1.4 Contamination control procedure |  |
| HP 1.5 Glass and brittle plastic control procedure |  |
| HP 1.6 Blade control procedure |  |
| HP 1.7 Cleaning procedure |  |
| HP 1.8 Pest control procedure |  |
| HP 1.9 Food contact compliance procedure |  |
| HP 1.10 Product recall procedure |  |
| HP 1.11 Emergency procedure |  |
| HP 1.12 Microbiological audit procedure |  |
| HP 1.13 GMP / GHP audit procedure |  |
| HP 1.14 Hygiene review procedure |  |
| HP 1.15 Traceability plan |  |

PAGE 8 OF 8									SOP 1.1 Revision 5

Issue date: 19.11.02