INSTRUCTIONS FOR INCIDENT MANAGER (DEPT HEAD, CHARGEHAND OR SUPERVISOR): RESPONSIBILITY 1. It is the responsibility of the Department Head, Chargehand or Supervisor who is initially notified of a contamination incident to assume the role of ‘Incident Manager’ and to ensure that the instructions detailed below are carried out. RECORDING 1. A complete record must be kept on the reverse of this document of all actions taken from the moment a contamination incident is discovered until it is deemed safe to recommence production and Management are satisfied that no contaminated product is likely to be despatched. IMMEDIATE ACTION 1. In the event of contamination being discovered, stop production within a radius of 10 metres (33.3 feet) of the incident. 2. None of the product (packed or exposed) or packaging within the 10 metre radius must be moved. SUBSEQUENT ACTION 1. Cordon off the 10 metre ‘zone of risk’ with tape. 2. Any product or packaging that might possibly have been contaminated must be removed and quarantined prior to a very thorough inspection or disposal, as appropriate. Nothing must be released for use/despatch if freedom from contamination cannot be guaranteed. 3. When contamination is not discovered until some time after it occurred, all products to have passed through the 10 metre radius must be considered at risk. If the Department Head believes that contaminated product may have been despatched to the customer then under the instruction of a Director the Product recall procedure HP 1.10 may be instigated. CLEAN UP 1. The area must be thoroughly cleaned using relevant tools (dustpan, brush, vacuum) and cleaning chemicals as required. 2. Any debris must be placed in a bag and sealed before they are removed from the production area. 3. The dustpan, brush, vacuum nozzles, etc, must be placed in a spare bag and sealed before they are removed from the production area for inspection and cleaning. 4. It is recommended that personnel involved in the clean up operation change overalls and clean their footwear before re-entering any production areas. FINAL ACTION: 1. On completion of all actions this document must be taken to the Technical Department. INCIDENT MANAGER COMPLETE ALL DETAILS OVERLEAF

| DATE OF INCIDENT | TIME OF INCIDENT | INCIDENT MANAGER | SIGNATURE |
| --- | --- | --- | --- |
| DESCRIPTION OF INCIDENT: (INCLUDE WORKS ORDER NUMBERS OF JOBS WITHIN THE ‘ZONE OF RISK’) |  |  |  |
| DESCRIPTION OF ACTION TAKEN: |  |  |  |