| INSTRUCTIONS FOR INCIDENT MANAGER (DEPT HEAD, CHARGEHAND OR SUPERVISOR): RESPONSIBILITY 1. It is the responsibility of the Department Head, Chargehand or Supervisor who is initially notified of a glass/plastic breakage to assume the role of ‘Incident Manager’ and to ensure that the instructions detailed below are carried out. RECORDING 1. A complete record must be kept on the reverse of this document of all actions taken from the moment a breakage is discovered until it is deemed safe to recommence production and Management are satisfied that no contaminated product is likely to be despatched. IMMEDIATE ACTION 1. In the event of a glass/plastic breakage being discovered, stop production within a radius of 10 metres (33.3 feet) of the incident. 2. None of the product (packed or exposed) or packaging within the 10 metre radius must be moved. SUBSEQUENT ACTION 1. Cordon off the 10 metre ‘zone of risk’ with tape and organise a very thorough search of the area at all possible levels where fragments may have lodged. 2. If fragments are discovered close to the perimeter of the zone the search should be extended. When the extent of the spread of fragments is determined, the cordon can be repositioned and normal working practices resumed outside the cordoned area if it is considered safe to do so. 3. Any product or packaging that might possibly have been contaminated must be removed and quarantined prior to a very thorough inspection or disposal, as appropriate. Nothing must be released for use/despatch if freedom from contamination cannot be guaranteed. 4. When damage to a glass/plastic item is not discovered until some time after the breakage occurred, all product to have passed through the 10 metre radius since the previous recorded glass audit (see Technical Department for details) must be considered at risk. If the Department Head believes that contaminated product may been despatched to the customer then under the instruction of a Director the Product recall procedure HP 1.10 may be instigated. CLEAN UP 1. Any broken glass/plastic still fixed to a damaged piece of equipment or window must first be carefully removed and placed in a heavy-duty bag. If possible remove the whole component and seal in a bag, then fit a replacement. 2. Carefully collect every fragment of glass/plastic into a heavy-duty bag. Large pieces may be picked up by gloved hands or by the use of the dustpan and brush. 3. Smaller pieces should be vacuumed or carefully swept up (brushes can sometimes flick fragments to one side). Do not omit high level surfaces. 4. When all fragments appear to have been collected and bagged the incident Manager will carry out a final thorough inspection. If no fragments are found the bags must be sealed and immediately placed in the rubbish skip. 5. The dustpan, brush, vacuum nozzles, etc must be placed in a spare bag and sealed before they are removed from the production area for inspection and cleaning. 6. It is recommended that personnel involved in the clean up operation change overalls and clean their footwear before re-entering any production areas. FINAL ACTION: 1. On completion of all actions this document must be taken to the Technical Department. 2. The ‘glass breakage kit’ must be restocked - see Technical Department. INCIDENT MANAGER COMPLETE ALL DETAILS OVERLEAF |  |  |  |
| --- | --- | --- | --- |
| DATE OF INCIDENT | TIME OF INCIDENT | INCIDENT MANAGER | SIGNATURE |
| DESCRIPTION OF INCIDENT: (INCLUDE PRODUCTION ORDER NUMBERS OF JOBS WITHIN THE ‘ZONE OF RISK’) |  |  |  |
| DESCRIPTION OF ACTION TAKEN: |  |  |  |